Course Code & Course Name: ITA05 & COMPUTER VISION

| Assignment Title | A smart-city surveillance company is developing a real-time computer vision system for detecting objects through roadside cameras. |
| --- | --- |
| Course Outcome(s)- CO: | CO2: Apply image enhancement and segmentation techniques for extracting meaningful regions from images. CO3: Analyse and extract image features using detection and matching techniques for effective image representation and comparison CO4: Compare object and pattern recognition techniques for interpreting visual data with spatial and motion characteristics. |
| Bloom's Taxonomy Level | BL4 – Analyze, BL5 – Evaluate |
| SDG Mapping (SDG 1-17) | SDG 9 – Industry, Innovation and Infrastructure; SDG 11 – Sustainable Cities and Communities |
| Problem Statement: A smart-city surveillance company is developing a real-time system for detecting objects through roadside cameras. The deployed solution must have latency ≤ 100 ms, accuracy ≥ 85%, and operate using moderate computational resources(An edge device such as an NVIDIA Jetson Nano / Jetson Xavier NX (typical roadside-AI hardware) ). A city surveillance system must operate under the following conditions: ● If latency > 100 ms → system fails ● If accuracy < 85% → unacceptable ● Hardware supports only moderate computation The engineering team is evaluating three models: Viola-Jones (78% accuracy, low latency), YOLO (92% accuracy, moderate latency), and Deep Learning model (95% accuracy, high latency). As the AI engineer, critically evaluate the suitability of each model against the operational constraints, determine the most appropriate model for deployment, and justify your decision using logical reasoning, performance trade-offs, and real-world surveillance requirements. Assessment Rubrics Criteria (CO Mapping); Max Marks; Excellent; Good; Satisfactory; Needs Improvement. Problem Understanding & Computer Vision Fundamentals (CO1); 15; Clearly identifies the industrial defect-detection problem, accuracy requirement (>95%), product variations, and role of computer vision in automated inspection.; Identifies most requirements with minor gaps and demonstrates good understanding of the problem.; Basic understanding of defect detection is demonstrated, but some requirements are unclear.; Problem requirements are misunderstood or incorrectly interpreted.. Image Preprocessing, Enhancement & Feature Analysis (CO2); 20; Appropriately proposes preprocessing such as resizing, noise removal, illumination correction, filtering, segmentation, and relevant feature extraction for defect analysis.; Appropriate preprocessing and feature extraction techniques are selected with minor gaps.; Basic preprocessing techniques are identified but their application is limited.; Preprocessing/feature extraction techniques are inappropriate or missing.. Comparison of Traditional and Deep Learning Approaches (CO3/CO4); 20; Provides a comprehensive comparison based on accuracy, adaptability, computational cost, training requirements, robustness, scalability, and deployment suitability.; Provides a relevant comparison covering most important factors.; Provides a basic comparison with limited analysis of performance and adaptability.; Comparison is incomplete, inaccurate, or lacks technical reasoning.. Solution Design & Model Selection (CO4/CO5); 15; Designs a suitable end-to-end defect detection pipeline and selects deep learning with strong justification for achieving >95% accuracy and adapting to product variations. Clearly addresses training, validation, and deployment.; Appropriate solution and model are selected with reasonable justification.; Solution is partially developed or model selection has limited justification.; No suitable solution design or model selection is provided.. Analysis, Evaluation & Justification (CO4); 10; Critically evaluates the performance–adaptability trade-off and convincingly justifies the selected approach using industrial requirements and practical deployment considerations.; Provides good analysis and justification with minor limitations.; Provides general analysis but limited critical evaluation.; Little or no meaningful analysis or justification.. Implementation, Results & Visualization (CO5); 10; Successfully implements the proposed approach using appropriate computer vision tools; presents meaningful results, metrics, sample detections, and visualizations.; Implementation works with minor issues and provides relevant results.; Partial implementation with limited or inconsistent results.; Implementation is missing, non-functional, or results are not demonstrated.. Reflection, Industrial Relevance & SDG Mapping; 10; Provides thoughtful reflection on design decisions, challenges, learning outcomes, industrial quality inspection, and clear SDG relevance.; Provides reasonable reflection and SDG connection.; Reflection is present but generic.; Reflection is missing or lacks meaningful connection to the problem.. |

# 1. Problem Understanding

A smart-city surveillance company is developing a real-time computer vision system for detecting objects through roadside cameras. The deployed solution must run on an edge device — such as an NVIDIA Jetson Nano or Jetson Xavier NX (typical roadside-AI hardware) — and satisfy three operating conditions simultaneously:

- If latency > 100 ms → the system is considered failed.

- If accuracy < 85% → the result is unacceptable.

- The hardware supports only moderate computation — no server-class GPU is available at the roadside.

The engineering team is evaluating three candidate models: Viola-Jones (78% accuracy, low latency), YOLO (92% accuracy, moderate latency), and a heavy Deep Learning model (95% accuracy, high latency). As the AI engineer, the task is to critically evaluate each model against these operational constraints, determine the most appropriate model for deployment, and justify the decision using logical reasoning, performance trade-offs, and real-world surveillance requirements.

This is fundamentally a constraint-satisfaction problem, not a “pick the highest accuracy” problem: a model that cannot meet the real-time latency budget is operationally useless for a live camera feed, no matter how accurate it is offline.

# 2. Functional and Non-Functional Requirements

| Requirement | Type | Description |
| --- | --- | --- |
| Latency ≤ 100 ms | Non-functional (hard constraint) | End-to-end inference time per frame must not exceed 100 ms, or the system is considered failed for real-time roadside use. |
| Accuracy ≥ 85% | Non-functional (hard constraint) | Detection accuracy must be at least 85%, or the result is unacceptable for a safety-relevant surveillance application. |
| Moderate compute only | Non-functional (hardware constraint) | Must run on edge hardware such as an NVIDIA Jetson Nano / Xavier NX — no dedicated server-class GPU available at the roadside. |
| Object detection | Functional | Detect and localize relevant road objects (vehicles, pedestrians, signage) frame by frame from a live camera feed. |
| Continuous operation | Functional | Must sustain real-time throughput on a continuous video stream, not just single-image inference. |

# 3. Image Preprocessing, Enhancement & Feature Analysis

Before detection, each frame from the roadside camera feed is processed as follows:

- Resizing / normalization — fixed input resolution for consistent, predictable inference time.

- Noise removal — Gaussian / bilateral filtering to handle rain, motion blur, and sensor noise.

- Illumination correction — histogram equalization / CLAHE to handle day-night transitions and headlight glare.

- Edge and contrast enhancement — Sobel / Canny operators to sharpen structural features.

- Background separation — isolating moving foreground objects (vehicles, pedestrians) from the static road background to reduce the search space and speed up detection.

- Feature extraction — HOG-style handcrafted features for the classical (Viola-Jones) pipeline vs. learned convolutional features for YOLO / deep models.

# 4. Comparison of Candidate Models

Each candidate is scored against the two hard numeric constraints (latency ≤ 100 ms, accuracy ≥ 85%) and the hardware's moderate-compute ceiling.

| Model | Accuracy | Latency | Compute | Verdict |
| --- | --- | --- | --- | --- |
| Viola-Jones | 78% | Low | Very low | REJECTED – accuracy |
| YOLO (v8n) | 92% | Moderate | Moderate | DEPLOYABLE |
| Deep Learning (2-stage) | 95% | High | High | REJECTED – latency & compute |

Additional qualitative comparison factors:

- Robustness to lighting, angle and occlusion: Viola-Jones – poor; YOLO – good; Deep Learning – best.

- Multi-class scalability: Viola-Jones is largely limited to single-class detection (e.g., faces); YOLO and deep models natively support multi-class, multi-object detection needed for a road scene (vehicles, pedestrians, signage).

- Edge-deployment suitability: YOLO is purpose-built for real-time, edge-optimized inference (e.g., via TensorRT); two-stage deep detectors generally require GPU resources beyond what moderate edge hardware provides unless heavily pruned or quantized.

# 5. Selected Approach and Justification

Selected model: YOLO (a lightweight, edge-optimized variant such as YOLOv8n / YOLO-tiny).

Constraint-based reasoning:

- Viola-Jones — accuracy = 78% < 85% required → REJECTED outright, regardless of its excellent latency. Speed cannot compensate for an unacceptable detection rate in a safety-relevant application.

- Deep Learning (2-stage) — accuracy = 95% (highest of the three) but latency is high and, on moderate edge compute, is expected to exceed the 100 ms real-time budget → REJECTED. A highly accurate model that cannot keep pace with the live camera frame rate is operationally useless for continuous surveillance.

- YOLO — accuracy = 92% ≥ 85% AND latency is moderate, and can be tuned to remain ≤ 100 ms on Jetson-class hardware (e.g., with TensorRT INT8 optimization) → ACCEPTED. It is the only candidate that satisfies both hard constraints simultaneously while matching the compute profile of the target hardware.

Real-world justification: roadside surveillance needs sustained, real-time throughput far more than it needs peak offline accuracy. A slightly-less-accurate model that never misses its time budget is more valuable operationally than a highly accurate model that periodically stalls or drops frames — which, in a traffic-safety context, translates directly into missed detections in real time regardless of the model's benchmark accuracy.

# 6. Algorithm / Pseudocode

## 6.1 Model Selection Logic

BEGIN SelectModel(candidates, LATENCY_LIMIT = 100 ms, ACCURACY_LIMIT = 85%)

FOR EACH model IN candidates:

reasons <- empty list

ok <- TRUE

IF model.latency > LATENCY_LIMIT:

ok <- FALSE

reasons.append("latency exceeds limit -> system fails")

IF model.accuracy < ACCURACY_LIMIT:

ok <- FALSE

reasons.append("accuracy below limit -> unacceptable")

IF model.compute == "high":

ok <- FALSE

reasons.append("exceeds moderate edge-device budget")

model.verdict <- ok ? "DEPLOYABLE" : "REJECTED"

model.reasons <- reasons

deployable <- FILTER candidates WHERE verdict == "DEPLOYABLE"

RETURN deployable[0] IF deployable is not empty ELSE "No model qualifies"

END

## 6.2 End-to-End Detection Pipeline

BEGIN DetectionPipeline(frame)

frame <- Resize(frame, fixed_resolution)

frame <- DenoiseFilter(frame)                 // Gaussian / bilateral

frame <- IlluminationCorrect(frame)           // CLAHE / histogram eq.

roi   <- BackgroundSubtract(frame)            // isolate moving objects

t_start <- CurrentTime()

detections <- YOLO_Infer(roi)                 // forward pass

detections <- NonMaxSuppression(detections)

t_latency <- CurrentTime() - t_start

IF t_latency > LATENCY_LIMIT:

LogFrameDropped()

ELSE:

DrawBoundingBoxes(frame, detections)

EmitToDashboard(frame, detections, t_latency)

RETURN detections, t_latency

END

# 7. Reflection and SDG Relevance

- SDG 9 — Industry, Innovation and Infrastructure: demonstrates efficient edge-AI deployment that does not depend on constant cloud connectivity, supporting resilient, innovative city infrastructure.

- SDG 11 — Sustainable Cities and Communities: real-time roadside object detection directly supports traffic safety, pedestrian protection, and congestion management — core needs of sustainable urban mobility.

The key engineering takeaway from this exercise is that model selection under real-world deployment constraints is a trade-off problem, not an accuracy-maximization problem — the ability to reason about latency, compute, and accuracy jointly is what separates a lab-grade model from a deployable one.

SOURCE CODE:

"""

Flask backend for the Smart-City Roadside Object Detection dashboard.

Routes:

GET  /                 -> dashboard UI

POST /api/detect       -> run YOLO + Viola-Jones on an uploaded image,

return annotated images (as base64) + metrics

GET  /api/compare      -> run the fixed 3-model constraint comparison

(Viola-Jones / YOLO / Deep Learning) and

return the verdicts + chart

"""

import base64

import io

import time

from pathlib import Path

import cv2

import numpy as np

from flask import Flask, jsonify, render_template, request, send_from_directory

from ultralytics import YOLO

BASE_DIR = Path(__file__).parent

UPLOAD_DIR = BASE_DIR / "uploads"

UPLOAD_DIR.mkdir(exist_ok=True)

app = Flask(__name__)

# Load YOLO once at startup (expensive to reload per-request)

yolo_model = YOLO("yolov8n.pt")

vj_cascade = cv2.CascadeClassifier(cv2.data.haarcascades + "haarcascade_fullbody.xml")

LATENCY_LIMIT_MS = 100

ACCURACY_LIMIT = 0.85

# Fixed, literature/assignment-given accuracy figures for the 3-model comparison

KNOWN_ACCURACY = {

"Viola-Jones": 0.78,

"YOLO (v8n)": 0.92,

"Deep Learning (2-stage)": 0.95,

}

def image_to_base64(img_bgr):

"""Encode an OpenCV BGR image as a base64 PNG data URL."""

ok, buf = cv2.imencode(".png", img_bgr)

if not ok:

return None

b64 = base64.b64encode(buf.tobytes()).decode("utf-8")

return f"data:image/png;base64,{b64}"

def run_viola_jones(img_bgr, n_runs=5):

gray = cv2.cvtColor(img_bgr, cv2.COLOR_BGR2GRAY)

vj_cascade.detectMultiScale(gray, scaleFactor=1.1, minNeighbors=4)  # warm-up

latencies = []

boxes = []

for _ in range(n_runs):

start = time.perf_counter()

boxes = vj_cascade.detectMultiScale(gray, scaleFactor=1.1, minNeighbors=4)

latencies.append((time.perf_counter() - start) * 1000)

vis = img_bgr.copy()

for (x, y, w, h) in boxes:

cv2.rectangle(vis, (x, y), (x + w, y + h), (0, 0, 255), 2)

return {

"latency_ms": round(float(np.mean(latencies)), 2),

"detections": int(len(boxes)),

"image": image_to_base64(vis),

}

def run_yolo(img_bgr, n_runs=3):

yolo_model.predict(source=img_bgr, verbose=False)  # warm-up

latencies = []

results = None

for _ in range(n_runs):

start = time.perf_counter()

results = yolo_model.predict(source=img_bgr, verbose=False)

latencies.append((time.perf_counter() - start) * 1000)

r = results[0]

confs = r.boxes.conf.cpu().numpy() if r.boxes is not None else np.array([])

annotated = r.plot()

return {

"latency_ms": round(float(np.mean(latencies)), 2),

"detections": int(len(confs)),

"avg_confidence": round(float(np.mean(confs)), 3) if len(confs) else 0.0,

"image": image_to_base64(annotated),

}

def check_constraints(name, accuracy, latency_ms, compute):

reasons = []

ok = True

if latency_ms > LATENCY_LIMIT_MS:

ok = False

reasons.append(f"latency {latency_ms:.1f} ms > {LATENCY_LIMIT_MS} ms limit \u2192 system fails")

if accuracy < ACCURACY_LIMIT:

ok = False

reasons.append(f"accuracy {accuracy*100:.1f}% < {ACCURACY_LIMIT*100:.0f}% required \u2192 unacceptable")

if compute == "high":

ok = False

reasons.append("requires high compute \u2192 exceeds moderate edge-device budget")

return {

"model": name,

"accuracy_pct": round(accuracy * 100, 1),

"latency_ms": round(latency_ms, 2),

"compute": compute,

"verdict": "DEPLOYABLE" if ok else "REJECTED",

"reasons": reasons or ["meets all constraints"],

}

@app.route("/")

def index():

return render_template("index.html")

@app.route("/images/<path:filename>")

def serve_sample_image(filename):

return send_from_directory(BASE_DIR / "images", filename)

@app.route("/api/detect", methods=["POST"])

def api_detect():

if "image" not in request.files:

return jsonify({"error": "No image uploaded"}), 400

file = request.files["image"]

file_bytes = np.frombuffer(file.read(), np.uint8)

img_bgr = cv2.imdecode(file_bytes, cv2.IMREAD_COLOR)

if img_bgr is None:

return jsonify({"error": "Could not decode image"}), 400

vj_result = run_viola_jones(img_bgr)

yolo_result = run_yolo(img_bgr)

vj_verdict = check_constraints("Viola-Jones", KNOWN_ACCURACY["Viola-Jones"], vj_result["latency_ms"], "low")

yolo_verdict = check_constraints("YOLO (v8n)", KNOWN_ACCURACY["YOLO (v8n)"], yolo_result["latency_ms"], "moderate")

return jsonify({

"viola_jones": {**vj_result, **vj_verdict},

"yolo": {**yolo_result, **yolo_verdict},

})

@app.route("/api/compare")

def api_compare():

"""Runs the fixed 3-model comparison on a bundled sample image."""

sample_path = BASE_DIR / "images" / "bus.jpg"

img_bgr = cv2.imread(str(sample_path))

vj_result = run_viola_jones(img_bgr)

yolo_result = run_yolo(img_bgr)

dl_latency = yolo_result["latency_ms"] * 5.0  # modeled: 2-stage ~5x slower than single-shot

results = [

check_constraints("Viola-Jones", KNOWN_ACCURACY["Viola-Jones"], vj_result["latency_ms"], "low"),

check_constraints("YOLO (v8n)", KNOWN_ACCURACY["YOLO (v8n)"], yolo_result["latency_ms"], "moderate"),

check_constraints("Deep Learning (2-stage)", KNOWN_ACCURACY["Deep Learning (2-stage)"], dl_latency, "high"),

]

deployable = [r for r in results if r["verdict"] == "DEPLOYABLE"]

return jsonify({

"constraints": {"latency_limit_ms": LATENCY_LIMIT_MS, "accuracy_limit_pct": ACCURACY_LIMIT * 100},

"results": results,

"selected_model": deployable[0]["model"] if deployable else None,

"sample_images": {

"viola_jones": vj_result["image"],

"yolo": yolo_result["image"],

},

})

if __name__ == "__main__":

app.run(debug=True, host="0.0.0.0", port=5000)

OUTPUT:

Assessment Rubrics

| Criteria (CO Mapping) | Max Marks | Excellent | Good | Satisfactory | Needs Improvement |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding & Computer Vision Fundamentals (CO1) | 15 | Clearly identifies the industrial defect-detection problem, accuracy requirement (>95%), product variations, and role of computer vision in automated inspection. | Identifies most requirements with minor gaps and demonstrates good understanding of the problem. | Basic understanding of defect detection is demonstrated, but some requirements are unclear. | Problem requirements are misunderstood or incorrectly interpreted. |
| Image Preprocessing, Enhancement & Feature Analysis (CO2) | 20 | Appropriately proposes preprocessing such as resizing, noise removal, illumination correction, filtering, segmentation, and relevant feature extraction for defect analysis. | Appropriate preprocessing and feature extraction techniques are selected with minor gaps. | Basic preprocessing techniques are identified but their application is limited. | Preprocessing/feature extraction techniques are inappropriate or missing. |
| Comparison of Traditional and Deep Learning Approaches (CO3/CO4) | 20 | Provides a comprehensive comparison based on accuracy, adaptability, computational cost, training requirements, robustness, scalability, and deployment suitability. | Provides a relevant comparison covering most important factors. | Provides a basic comparison with limited analysis of performance and adaptability. | Comparison is incomplete, inaccurate, or lacks technical reasoning. |
| Solution Design & Model Selection (CO4/CO5) | 15 | Designs a suitable end-to-end defect detection pipeline and selects deep learning with strong justification for achieving >95% accuracy and adapting to product variations. Clearly addresses training, validation, and deployment. | Appropriate solution and model are selected with reasonable justification. | Solution is partially developed or model selection has limited justification. | No suitable solution design or model selection is provided. |
| Analysis, Evaluation & Justification (CO4) | 10 | Critically evaluates the performance–adaptability trade-off and convincingly justifies the selected approach using industrial requirements and practical deployment considerations. | Provides good analysis and justification with minor limitations. | Provides general analysis but limited critical evaluation. | Little or no meaningful analysis or justification. |
| Implementation, Results & Visualization (CO5) | 10 | Successfully implements the proposed approach using appropriate computer vision tools; presents meaningful results, metrics, sample detections, and visualizations. | Implementation works with minor issues and provides relevant results. | Partial implementation with limited or inconsistent results. | Implementation is missing, non-functional, or results are not demonstrated. |
| Reflection, Industrial Relevance & SDG Mapping | 10 | Provides thoughtful reflection on design decisions, challenges, learning outcomes, industrial quality inspection, and clear SDG relevance. | Provides reasonable reflection and SDG connection. | Reflection is present but generic. | Reflection is missing or lacks meaningful connection to the problem. |

Deliverables

1. Problem Analysis and Pseudocode – Include problem understanding, functional requirements, comparison, selected approach, justification, and algorithm/pseudocode.

2. Implementation & Results – Include source code, dataset description, preprocessing, detection/classification, training/testing where applicable, evaluation metrics, visualizations, sample detections, and comparative analysis.

3. GitHub Upload – Include source code, pseudocode, dataset information/source, README, installation/execution instructions, results, performance evaluation, and project documentation. GitHub repository link must be submitted with the assignment.